Project: Titan TV 2K 2026

AQ fine tuning

XML-XLS converter. Project Task

Application version 2.x

Prepared by: Alex Jermakov

Created: 2026 05 19

Updated: 2026 05 25

Scope: this document describes project task conditions, required for XML-XLS converter application to be created by Codex AI tool.

Note: This is project task modification for application version 2.x, which is simplified against the project task version 1.x, with target to use Copy+Past operation instead of full function app. Such decision must simplify application development. There are applied following modifications:

- Removed TV model selection

- Removed MAP file (it is used now internal file in the project task)

- Removed input XLS file

- Defined how to identify Profile in the XML file

Project Task

Create certain application under conditions described below.

General information

Application name:	Sharp Titan TV. AQ. XML-XLS converter

Version:		2.0

Application type:	web online application

Application URL:	new one, different to the application of Version 1.x

Functionality:		to analyze audio data inside of XML file and create data for manual Copy+Paste operation.

Descriptions:

- XML file is generated by Dolby Tuning Tool and it contains Dolby Audio Processing (DAP) settings. It is input file for the converter

- XLS output file must contain DAP settings in the format, acceptable for the manual implementation to XLS file by Copy&Paste operation

- Dependencies between data in XML file and in XLS files are described in the XML-XLS Dependencies file (named as MAP2.0).

General functionality

XML file description

In the XML file there must be identified the settings dedicated to internal speaker. These settings are located inside of the text portions, which starts with a string :

<endpoint type="internal_speaker" brand="Brand" model="TVMOdelName">.

And ending with a string:

</endpoint>

Note: Values “Brand” and “TV ModelName” may variate, they have no sense for identification.

There is one more endpoint defined by string <endpoint type="headphones" brand="user-editable" model="user-editable">, which is dedicated for headphones.

The application must operate only with data developed for internal speakers. Data for headphones must be out of scope.

In the XML file there is also important to identify the DAP audio settings, dedicated to different Sound Profiles.

DAP settings for every sound profile are located inside of the text portion, which is starting with string

<profile type="ProfileName" name="txtProfileName">

And it is ending with the string:

</profile>

Full list of all sound Profiled is shown in the table below. Profiles Game, Night and Off are excluded from the application scope.

| Profile Name in XML file | Supported by application |
| --- | --- |
| Movie | yes |
| Music | yes |
| Game | no |
| Night | no |
| Voice | yes |
| User Selectable | yes |
| Off | no |

MAP file

MAP file is file in XLSX format, which describe dependencies in between Sound Profiles and Sound parameter names in between XML and XLS files.

This file is the internal project file, which can be updated by developer, but it must be used for application built.

The file name is defined as MAP2.0.xlsx.

Title of this file is demonstrated below:

Output XLS file

Output file must must be prepared in XLSX format and it must contain the data from XLM file and relevant data, generated for Copy&Paste operation.

Title for this file is shown below. It is the same as for MAP file.

Parameters values must be found in the XML file. They are defined in Decimal format. Application must convert them to Hexadecimal format and put the value to the corresponding column named “GAIN(HEX)”.

To provide better end user experience, let add to the XLS file some title information like:

Application name:	Sharp Titan TV. AQ. XML-XLS converter

Version:		2.0

Conversion date:	<actual date and time>

Input file name:		<XML file name>

Output file name: 	<XLS file name>

Note:			Use the manual Copy&Paste operation by following steps:

- Select required Sound Mode in the column D (named as DSP)

- Select whole data from the column “GAIN(HEX)”

- Open AQ.XLS file

- Filter corresponding sound mode in the AQ.xls file

- Select required TV model in the column

- Paste selected values to this column

- Repeat steps 5-6 for other TV models, if required

LOG file

LOG file must indicate internal operations of the application. Log data must be able to be collected in output LOG file.

User interface

User interface contains several windows, distributed on different screen location:

1.Title window with name of the application, version number, released date. It is located on the top of the screen.

3. Input window for XML file, located on the left side. It must support drag and drop technology

6. Output window for XLSX file. Located on the right side. It must support drag and drop technology.

8. Button “Convert”, located on the bottom of the screen, in the center

9.  Button “Download output XLSX file”, located on the bottom of the screen, on the right. If must be enabled, if output file is generated successfully.

10. Button “Download Log”. Located on the bottom.

11. Progress bar indicator of the conversion process, located on the screen bottom

12. Window for application error, status and debug messages, on the screen bottom