Project: Titan TV 2K 2026

AQ fine tuning

XML-XLS converter. Project Task

Prepared by: Alex Jermakov

Created: 2026 05 19

Updated: 2026 05 22b

Scope: this document describes project task conditions, required for XML-XLS converter application to be created by Codex AI tool.

Project Task

Create certain application under conditions described below.

General information

Application name:	Sharp Titan TV. AQ. XML-XLS converter

Version:		1.1b

Application type:	web online application

Functionality:		to update content of XLS file with the data extracted from the XML file

Descriptions:

- XML file is generated by Dolby Tuning Tool and it contains Dolby Audio Processing settings. It is input file for the converter

- XLS file contains Audio settings for TitanOS TV. These settings will be used for implementation to TitanOS TV. It is output file for converter

- MAP file, which is used for definition of the links between data in XML file and data in XLS file,

- LOG file, which contains a list of the modification applied to the XLS file

Functionality

- Converter application must fill out GAIN columns in XLS file worksheet “AQ_Tbl”, which have numbers listed below:

- Column number E (dedicated to TV model 1T-C24JF2x55E(K)B)

- Column number H (dedicated to TV models T-C32JF2x55E(K)B, 1T-C32JF3x55E(K)B, 1T-C32JF3x55E(K)B, 2T-C32JF2x55E(K)B)

- Column number K (dedicated to TV models 2T-C40JF2x55KE(K)B, 2T-C40JF3x55KE(K)B)

- Column number N  (dedicated to TV models 2T-C43JF2x55E(K)B)

Columns names cells are located in the line 25 . Data in all columns is stored starting from line 26 onward.

Data in XLS file must be updated exclusively for the columns defined by user in the window: TV model selection.

Log management

During conversion there must be generated LOG file.

In the LOG window there must be collected following information for every data modified in the XLS file:

Name of the item from column A which has name DSP

Name of the item from column B which has name Description

Name of the item from column C which has name Parameters

Previous Value from the columns F, or I, or L, or O, which have name Gain (HEX)

New Value, which is applied to the column F, or I, or L, or O.

LOG file must be able to be downloaded in XLS format.

- Mapping the data in the MAP file is located in the worksheet “MAP” and it is arranged in such way:

- Item name for XML file is defined in the column 4 “Mapped to DAP XML”

- Converter must take this name from every line, one by one, and search it in XML file

- If such item name is obtained in the XML file, then data in the GAINS columns must be replaced with the data extracted from XML file

- There is possible to obtain several items in XML file with the same name

- At this case there must be taken in account the data in the column DSP and column Description. Data replacement must be executed only in case, when all three names match.

- If the item is marked as “Not relevant” or cell is just empty, then in must be excluded from the search.

User interface

User interface contains several windows, distributed on different screen location:

1.Title window with name of the application, version number, released date. It is located on the top of the screen.

2. Input window for AQ settings version number entering. It must not be empty. Located on the left side

3. Input window for XML file, located on the left side. It must support drag and drop technology

4. Input window for XML file, located on the left side. It must support drag and drop technology

5. Input window for MAP file. Located on the left side. It must support drag and drop technology. The window must not be empty.

6. Output window for XLS file. Located on the right side. It must support drag and drop technology. The window must not be empty.

7. Output window for LOG file. Located on the right side.

8. Button “Convert”, located on the bottom of the screen, in the center

. Button “Download converted XLS file”, located on the bottom of the screen, on the right. If must be enabled, if output file is generated successfully.

10. Button “Download Log”. Located on the bottom.

11. Progress bar indicator of the conversion process, located on the screen bottom

12. Window for application error, status and debug messages, on the screen bottom

13. Output Window “AQ settings for manual Copy&Paste” with generated column of the output GAIN data, which could be manually copied and pasted to the output XLS file. Located on the right side

14. Input window “TV model selection”. This window must let end user select TV model, for which he likes the settings to be modified during conversion. TV model names are defined in the columns E, H, K, N of the XLS file worksheet “AQ_Tbl”.

15. Tick mark to enable option “To apply the same AQ setting for 32”, 40”, 43” TV models”. This option must be active only if user selected TV models of sizes 32”, 40”, 43” in the window “TV model selection” (T-C32JF2x55E(K)B, 1T-C32JF3x55E(K)B, 1T-C32JF3x55E(K)B, 2T-C32JF2x55E(K)B), ls 2T-C40JF2x55KE(K)B, 2T-C40JF3x55KE(K)B),  2T-C43JF2x55E(K)B)). If user enabled this option, then new GAINS values must be applied to all TV models of 32”, 40”, 43” sizes.

AQ settings version

AQ settings version number defined in the window “AQ settings version”, must be applied to the cell B2 of the output XLS file.

Default values

There must be proposed following default files names:

For XLS file: C:\Data\TVS\Projects\Titan\AQ\Tools\XML XLS converter\XLS.xls

For MAP file: C:\Data\TVS\Projects\Titan\AQ\Tools\XML XLS converter\MAP.xls

For XML file: C:\Data\TVS\Projects\Titan\AQ\Tools\XML XLS converter\XML.xml

For Version number the default value is: 1.0

Default values must be taken into account if user has not selected them and tried to click on button Convert

This is temporary option, to be used during application debugging. Later it will be removed.

XLS file

XLS file formatting is preferred to be kept after conversion. But if it is not possible, then it is acceptable to use output XLS file with no formatting.